Nuclear Safety and Inherent Self-protection Investigations for Modular High Temperature Gas-cooled Reactors (HTGR-M). Experimental Demonstration in the ASTRA Facility of the Inherent Safety Characteristics of HTR's

A.L. Balanin, A.A. Bobrov, V.F. Boyarinov, V.P. Garin, A.E. Glushkov, E.S. Glushkov, V.N Grebennik, M.I. Gurevich, S.B. Danelia, V.A. Kleymenov, G.V. Kompaniets, N.E. Kuharkin, V.A. Nevinitsa, D.N. Polyakov, N.N. Ponomarev-Stepnoy, A.A. Sedov, P.A. Fomichenko, E.I. Chunyaev

- Russian Research Centre «Kurchatov Institute», Moscow, Russia

e-mail: alb@dhtp.kiae.ru, sedov@dhtp.kiae.ru

Modular high temperature gas-cooled reactors (HTGR) with cores composed of graphite fuel elements being developed in a number of countries (USA, Japan, Russia, China, etc.) have improved safety and are promising for the next generation of nuclear power plants.

Peculiarities of HTGR physics such as feedbacks from core graphite moderator and kernel fuel temperature characterized by reactivity coefficients and effects play the most important role in inherent self-protection of HTGR. In the same time, these effects have been studied in benchmark critical experiments in a very limited extent. Thus, experimental simulation of temperature effects of reactivity and efficiency of control rods is an important part in substantiation of self-protection of such reactors.

In the framework of ISTC project #0685.2 “Nuclear Safety and Inherent Self-protection Investigations for Modular High Temperature Gas-cooled Reactors (HTGR-M). Experimental Demonstration in the ASTRA Facility of the Inherent Safety Characteristics of HTR's” in Russian Research Center “Kurchatov Institute” a concept of the use critical facility “ASTRA” for experimental measurements of reactivity temperature effects in HTGRs with annular cores has been developed.

Having the possibility to reproduce sizes of the annular core, inner, side, bottom and top reflectors in proportions typical for real reactor designs, “ASTRA” critical facility serves as a representative model of different Modular High-Temperature Gas-cooled Reactors.

A number of critical assemblies differing in configuration, size and composition of the core were investigated at the “ASTRA” critical facility at room temperature. These experimental investigations at room temperature allowed a lot of peculiarities of HTGR-M physics to be studied, and computation methods and codes to be verified for different HTGR designs.

At present time the “ASTRA” critical facility is prepared for conducting experiments on measuring of temperature effects of reactivity, criticality parameters and control rods efficiency under condition of heating to 600C.

In the present paper the concept of “Hot ASTRA” experiments is presented. General view of modified “ASTRA” critical facility is given together with new implemented systems and equipment, like electrical heating system, thermal insulation, inert gas supply system, side reflector cooling system and automatic information system.

Accompanying studies of neutron physics as well as thermal physics calculations in static and dynamic regimes of heating the “ASTRA” critical facility has been performed and main results of these researches are given in presented paper.