Use of high-performance moderators as neutron shielding in fast reactors

A.V.Varivtsev, I.Yu.Zhemkov and Yu.V.Naboischikov

(JSC “SSC RIAR”, Dimitrovgrad, Russia, www.niiar.ru)

A high damage power of high-energy neutrons is one of the main factors that limit service life of fast reactors. The high-energy neutron effect on irremovable components of the reactor (for example, reactor vessel) leads to their service life reduction. At the present time steel neutron shielding that is installed inside the reactor vessel is used to low fast neutron flux density and damage dose on the reactor vessel and other irremovable components.

This paper gives consideration to a possible use of high-performance moderators as a neutron shielding of fast reactors. It is clear that light nuclei-based moderator mounted in the side shield can soften neutron spectrum in shielding and irremovable components significantly and thus neutron damage power can be decreased significantly. Moreover, a neutron-absorption cross-section of structural materials, coolant and moderator itself increase as neutron energy decreases and thus the neutron flux density will decrease. So the moderator mounted in the side shield will significantly decrease a damage dose rate in the reactor vessel and extends its service life.

The materials incorporating light nuclei, for example hydrogen, deuterium and boron (the 11В isotope) can be chosen as a neutron moderator. One of the most efficient moderators of fast reactors is zirconium hydride that is being irradiated in the side shield of the BOR-60 reactor and has proved its high reliability degree during the long-term irradiation. Use of zirconium hydride as a neutron shielding decreases (by 3080%) the fast neutron flux density and damage dose on the reactor vessel significantly.

The results of examination made it possible to obtain useful model patent No 79210.