MODERN TRENDS IN RADIOACTIVE WASTES  PARTITIONING TECHNOLOGIES

V.A.Babain

# V.G. Khlopin Radium Institute

# www.khlopin.ru

28, 2nd Murinsky ave., 194021, St.-Petersburg, Russian Federation

Email: babain@khlopin.ru

Development of efficient technologies for recovery of long-lived radionuclides from high-level wastes (HLW) is urgent for implementation of the promising management methods (transmutation and disposal), as well as existing practice of HLW management. The composition of fractions therewith depends on method for further management of ecologically hazardous radionuclides, i.e. on their transmutation or production of stable matrices for their prolonged storage or final disposal.

The most important fractions generated by HLW reprocessing are cesium and strontium (in combination or separately) – on the one hand and actinides and lanthanides (in combination or separately) – on the other hand.

Among the different ways of HLW partitioning (precipitation, sorption, chromatography etc) of special interest are solvent extraction processes. The following extraction systems should be considered as most promising:

a) neutral organophosphorus compounds (alkylphosphine oxides ( TRPO and POR processes), carbamoylphosphine oxides, (TRUEX, “Russian”TRUEX,  SETFICS, ORGA processes),  phosphorylated calixarenes

b) acidic organophosphorus compounds (dialkylphosphoric acid - TALSPEAK, DIDPA  or CTH processes, zirconium salts of dialkylphosphoric acids  - ZIALEX process);

c) macrocyclic compounds (crown-ethers, calix-crowns – SREX, FPEX and other processes);

d) diamides (DIAMEX-process,  ARTIST-process)

e) hydrophobic anions in polar diluents (chlorinated cobalt dicarbollide – CCD in different diluents);

f) synergistic mixtures of different extractants (for example, UNEX-process).

As for now, the only way which has found commercial application is extraction of cesium and strontium by CCD – polyethylene glycol mixture in fluorinated diluent. This process was used at “Mayk” plant (Ozersk, Russia).

A number of promising reagents and processes have been developed during in past two decades. Great progress was demonstrated in implementation of crown ethers and calixarenes and their combination for Cs and Sr extraction. Mixture of crown ethers in fluorinated diluent were studied in Russia.  Calix-crowns were studied for Cs extraction in US and Europe and mixture calix-crown and crown ether was proposed for simultaneous Cs and Sr extraction (FPEX-process). In the same time – CCD + PEG in fluorinated diluent studied as alternative of FPEX process. It should be mentioned, that  fluorinated diluents or modifiers used in US and Russia to increase the  extractant solubility.

Tetraoctyldiamide of diglycolic acid (TODGA) actively studied as extractant not only zctinides, but strontium as well. Different diluents were proposed to increase te soplvate solubility.

A new group of amidic extractants receiving attention are diamides derivatives of dipicolinic acid. A significant synergistic effect was observed in the extraction of trivalent metals by different dipicolinamides (DPA) in the presence of CCD, and as such diamides of dipicolinic acid have been proposed instead of CMPO as components to be incorporated into a modified UNEX solvent.  Modification of UNEX process  flowsheet was also  proposed to separate nuclide in stripping stage.

Some other heterocyclic compounds were proposed as synergetic additives to CCD (see. Fig. 1). It was shown, that it’s possible to achieve the effective Am/Eu separation in acidic media. Most important, that these compounds are stable to hydrolysis.

| DPA | Dyp | ATP |
| --- | --- | --- |

Fig. 1 The structures of diamides of dipicolinic acid (DPA) and diamides of 2,2’-dipyridyl-6,6’-dicarboxylic acid (Dyp) 2,6-bis(1-aryl-1H-tetrazol-5-yl) pyridines (ATP) (R and R’ are H or alkyl).

Further efforts towards development of partitioning technologies are aimed at more efficient recovery of radionuclides from radwastes, higher safety of the process and equipment being in service and  waste minimization. Potential benefits and drawbacks of new processes are discussed.