Experimental capabilities of SSC RIAR research reactors in support of innovative reactor developments

I.Yu. Zhemkov, A.L. Izhutov, and M.N. Svyatkin

(JSC “SSC RIAR”, Dimitrovgrad, Russia, www.niiar.ru)

A rate of nuclear reactor commissioning and revitalization of work on innovative nuclear reactor development projects is expected to increase in Russia and throughout the world. To justify generation-IV nuclear reactors, it will be necessary to carry out experiments to prove physical parameters, safe operation, reliability, efficiency, and durability of structural materials and nuclear fuel characteristics. It is expedient to perform experiments in the research reactors. Only research reactors (RR) will allow reproducing the integrated effect of damaging factors on the reactor materials and reactor core components (neutron flux, temperature, corrosion attack and mechanical effect of coolant) to a great extent.

These days the major scope of research and development work in support of evolutionary and innovative development of nuclear-power engineering in Russia is performed in the research reactors: BOR-60, MIR.M1 and SM-3.

The main areas of the RIAR RR use are the following:

- Irradiation tests of structural, absorbing and moderating materials for a wide group of nuclear reactors including fusion reactors.

- Examination of typical and new types of nuclear fuel, achievement of high fuel burn-up;

- Investigation of fuel rod behavior providing for different types of nuclear fuel and nuclear fuel with burnable poisons, minor actinides, and regenerated nuclear fuel;

- Investigation of fuel rod and fuel assembly behavior under transient, power cycling and emergency conditions in support of their operational integrity in accordance with nuclear fuel operational integrity criteria;

- In-pile testing and closed nuclear fuel cycle studies, utilization of actinides and burning of long-lived fission products.

- Examination of new and modified coolants, their monitoring and quality management;

- Testing of new process equipment, instrumentation and diagnostic tools;

- Reactor physics study, post-irradiation examinations, thermal-hydraulic tests and other experiments to verify the computer codes;

- Applied researches (heat production, power production, radioisotope accumulation, modification work, neutron beam applications etc.).

Experimental capabilities of SSC RIAR research reactors can be widely used for international cooperation to perform irradiation tests of materials and reactor core components of generation-IV reactors.