- An approach to the quantitative evaluation of proliferation self-protection property

- of uranium-containing materials

- Kryuchkov E.F., Masterov S.V., Glebov V.B.*), Apse V.A., Shmelev A.N.

Moscow Engineering Physics Institute  (State University)

Kashirskoe shosse, 31, 115409, Moscow, Russia

*)  Phone: +7-(495)-323-92-42; FAX: +7-(495)-324-70-26; E-mail: VBGlebov@mephi.ru

Significance of nuclear materials (NM) consists mainly in their large energy potential in comparison with fossil organic fuel. The energy potential of NM may be realized through chain fission reaction (CFR) in sub-critical, critical and super-critical systems. The latter option has created so-called “non-proliferation problem”, that is a possibility for some NM (highly enriched uranium, for instance) to be used as a charge of nuclear explosion device (NED). That is why all the international innovative projects are planning to use NM with proliferation self-protection properties in advanced nuclear energy systems.

Wide-spread usage of enriched uranium in nuclear facilities increases the risk of uranium diversion for manufacturing of crude NED by sub-national terrorist groups. So, the simplest design of a hypothetical gun-type NED was taken as a geometrical model for evaluating the self-protection properties of uranium-containing NM. In the paper authors use probability concept of technical risk for evaluating NM self-protection levels. Using this concept  the self-protection levels of uranium  of different isotopic enrichment  are compared with self-protection of  20%-uranium. The approach developed by Mark, von Hippel and Lyman (*) was used to evaluate a relative nominal energy yield from CFR in the hypothetical gun-type NED. Necessary neutronic parameters were determined with application of Monte Carlo computer code MCNP.

The chains of illegal actions with diverted material, which lead to creation and application of NED are considered. The potential risk of NM illegal use for creation and application of  NED by a terrorist group is defined in the following way:

,

where  P – probability of NED creation and application;  D – potential damage from NED application.

The degree of counteraction to potential threat of illegal actions chain realization is expressed in the form of protection Z of material and is the quantity reversed to risk of NM illegal use:

Z=1/R   .

In conditions of action of internal safety barriers the quantity Z characterizes NM self-protection.

If potential damage from NED application to express by its energy yield (Y), it is possible to obtain simple approximation for NM self-protection:

Z(M) ≈ const×(M/Y(M))  ,

where M- mass of NM, Y- energy yield of NED.

The parameterized model of 	CFR propagation process was built for the systems conceptually similar to Little Boy. The results of model calculations for uranium of different enrichments demonstrated that:

-  Nominal energy yield for 20%-uranium  is an essential quantity and equal to 0,305 kt TNT. This value is 84 times less than for 90%-uranium;

- Specific energy yield drops down by a factor of about 2000 at transition from 90%-uranium to 20%-uranium. Nevertheless, specific energy yield from CFR in 20%-uranium remains higher by a factor about 200 than specific energy yield from conventional chemical explosives.

Numerical studies were carried out to determine NM proliferation self-protection at presence of internal neutron background. The algorithm of numerical studies was based on evaluating mean CFR energy yield with proper accounting for probability of premature CFR initiation. Proliferation self-protection of metal uranium was determined as a function of its isotopic enrichment and intensity of internal neutron source.

Neutron background in 20%-uranium is more intense (by a factor of 170) than that in 90%-uranium because of significant difference in total uranium mass and in mass of 238U.

The calculations demonstrated that, in the case of 90%-uranium, the probability of premature CFR initiation is rather low (Ppre =2.7%), and energy yield is practically equal to nominal energy yield. Quite different situation takes place for 20%-uranium. The probability of premature CFR initiation is very large (Ppre =98%), and energy yield may be defined by its mean value at premature CFR initiation. Dependence of relative uranium self-protection on its  enrichment  is obtained with accounting for internal neutron background. This dependence testify that the self-protection level of 90%-uranium is lower by a factor of about 13500 than that of 20%-uranium.